OSI TRACE 	  ( Fifth Book of OSI )

Code 1790-1FFF  ends 1F20. 1F21- 1FFF  nmenonics for disassembly.

Self modifying code ?

Calls 		( FFFC )  FBA9  FFEB

Locations	DF00

Execute 	@ 1790

----------------------------------------------------------------------

WinOSI :

Kernel=1=cegmonc1.rom ; MType = 0

Cold start D/C/W/M and enter Monitor with M

>L oad ... wait for MCLOAD window to open and select ExMon-0800.LOD

:L  oad     MCTrace.LOD  using Exmon checksum loader.

:G1790

-----------------------------------------------------------------------------------------------

Cursor initially at Program Counter  PC entry position.

Complete a four hex address will jump to command mode CMD ^

Commands:

<space> will execute in single step mode the instruction at the bottom line of the program counter window.

Tracing routines that use the keyboard may not work.

CC 		- Change the Program Counter address. Jumps to PC window.

G 		- Go will execute under trace control. Breakpoints encountered.

Bn 1-6 	- Breakpoint address. $FFFF  to clear. Jumps to break point window

J 		- Jump one instruction.

CA	CX	CY	CP - Change register contents. Changing the stack pointer CP overlapping its position of $28 will cause problems with the Trace program.

CS		 - Change the Status register to 0 or 1

0		- Disable CTRL

1		- Enable CTRL

CTRL - A	- Show command being executed

CTRL - C	- Show command being executed then BRK

N		- Restart in PC command mode

E		- Exit via Reset Vector

OSI TRACE  ( Fifth Book of OSI )

Subroutine Instructions:

W	- if the instruction at the PC position is a JSR the routine will be

executed as in G go command until a RTS is reached.

S		- as for Go until Stack Pointer overflows.

R		- RTS executed

Memory location changes:

M		- Memory address entry mode. Jumps to memory window.

0 - 9,A - F	- Address and Data entry

.		-  A dot in data mode after 4 digit address entered will return to

command level.

<SHFT @ >	- Ascii display

/		- Re open location

<SHFT : >	- Next address

< ^ SHFT N > - Previous address

CR		- new address